Forecast-Driven Energy-Aware Machine Scheduling for Sustainable Manufacturing Operations

Balaji Ramesh¹, Hemanth Kumar D², Bijlesh S³

Department of Artificial Intelligence and Data Science, Rajalakshmi Engineering College, Chennai, India

Email: { balaji.ramesh, hemanthkumar.d, bijlesh.s,}@rajalakshmi.edu.in

Abstract— Manufacturing facilities operate multiple energy-intensive machines under fluctuating electricity demand and time-varying tariffs, presenting a critical opportunity to curb energy expenditure while maintaining delivery performance. Conventional scheduling approaches such as First-Come-First-Served (FCFS) allocate production jobs solely according to arrival order and immediate machine availability, neglecting grid-level demand peaks and time-dependent energy costs. This study presents an integrated, data-driven framework for energy-aware machine scheduling that connects short-term industrial energy forecasting directly with production-job scheduling. Using a publicly available steel-industry energy dataset containing 35,040 observations at 15-minute intervals across an entire operational year, an XGBoost regression model is trained on temporal, cyclical, lagged, and rolling statistical features to forecast industrial energy demand across a 24-hour horizon (96 intervals). The forecasted energy profile is converted into a time-dependent cost profile and supplied to an FCFS-seeded deterministic local-search scheduler that considers machine compatibility, processing duration, job arrival, deadlines, machine non-overlap, and changeover requirements. In experimental evaluation, the proposed energy-aware schedule reduces modeled energy cost by 15.8% (from 1713.88 to 1443.78) and curtails peak-hour electrical load by 28.9% (from 1605.83 kWh to 1141.67 kWh), while maintaining a 22.5-hour makespan, a 57.92% machine utilization rate, zero tardiness, and 100% on-time completion. Concurrently, average job waiting time increases from 141.9 to 241.5 minutes (+99.6 minutes), quantifying the fundamental operational trade-off between energy efficiency and queueing latency in sustainable manufacturing.

Index Terms— Energy-aware scheduling, industrial energy forecasting, XGBoost, sustainable manufacturing, production scheduling, peak-load reduction.

I. INTRODUCTION

A. Background

Manufacturing industries operate a wide array of energy-intensive machines whose electricity consumption varies substantially according to equipment operating states, production throughput, time of day, and aggregate factory demand. In heavy and discrete manufacturing sectors, electrical power represents one of the largest controllable operational expenditures. Consequently, production scheduling is no longer merely a combinatorial assignment of jobs to processing resources to maximize throughput; it also directly governs the temporal distribution and magnitude of industrial power demand.

Traditional production scheduling paradigms primarily emphasize throughput-oriented metrics such as makespan minimization, total tardiness reduction, machine utilization maximization, or rigid adherence to job arrival sequences. However, modern sustainable manufacturing environments require operational decisions to explicitly consider time-varying electricity consumption and dynamic power tariffs. Energy-efficient scheduling has thus emerged as a pivotal pillar in industrial sustainability. Prior research has systematically classified energy-aware scheduling according to energy demand profiles, energy supply characteristics, and energetic coverage levels [1], [2].

Time-of-use (TOU) and dynamic electricity tariffs offer manufacturing facilities an economic incentive to shift flexible, non-critical production operations from expensive peak-demand intervals toward less costly off-peak periods. Literature reviews in energy-aware scheduling confirm that time-dependent energy costs can be embedded directly into scheduling objectives without violating underlying production constraints [3]. In parallel, machine-learning-based predictive analytics enables highly accurate estimation of future plant power demand from historical supervisory measurements. The UCI Steel Industry Energy Consumption dataset provides real-world electrical measurements collected from a steel manufacturing plant in South Korea containing 35,040 records at 15-minute resolution [4]. Initial studies on this dataset demonstrated the viability of data mining for industrial power prediction [5]. The present research unifies these two distinct domains by utilizing machine-learning-driven energy forecasting as an active, direct input to industrial machine scheduling.

B. Problem Statement

Industrial manufacturing facilities commonly operate heterogeneous machines in parallel to execute diverse customer work orders. When scheduling heuristics rely strictly on job arrival order or instantaneous machine availability (such as standard First-Come-First-Served dispatching), operations are frequently scheduled during peak electricity tariff windows or periods of intense plant-level power demand, even when relaxed job deadlines would easily permit temporal shifting to lower-cost windows.

This operational disconnect creates two severe inefficiencies:

Energy Cost and Peak Inefficiency: Production operations frequently coincide with high-cost peak periods, driving up modeled electricity expenditure and aggravating peak-demand charges on the electrical grid.

Process and Resource Inefficiency: Production jobs are assigned without joint optimization of machine power ratings, changeover durations, dynamic energy rates, and deadline slack, leading to suboptimal schedule configurations.

Therefore, the central research question addressed in this paper is: How can industrial energy-demand forecasting be systematically integrated with production-job scheduling to reduce modeled energy cost and peak-hour electrical load while strictly preserving machine compatibility, job delivery deadlines, and overall production throughput?

C. Motivation

The motivation for this investigation stems from the imperative for manufacturing enterprises to curb energy expenses and carbon footprints without sacrificing production volume or client delivery commitments. In industrial manufacturing, overall power consumption cannot be arbitrarily reduced simply by shutting down machinery, as production quotas must be fulfilled.

Instead, operational flexibility and deadline slack can be strategically exploited to reschedule flexible production jobs into energy-favorable time intervals. Highly accurate short-term energy forecasting provides predictive visibility into impending power demand, while an energy-aware scheduler can exploit this visibility to balance energy savings against operational latency. The proposed framework bridges the gap between predictive energy analytics and combinatorial scheduling, treating them as interconnected decision-making stages rather than isolated tasks.

D. Research Gap

Substantial literature exists in energy-efficient production scheduling, encompassing exact mathematical programming, metaheuristics, time-of-use tariff scheduling, and demand-response models [1]–[3], [6]. However, existing research exhibits a pronounced dichotomy:

Isolated Energy Forecasting: Energy forecasting studies predominantly focus on optimizing regression metrics (e.g., MAE, RMSE) on historical time series without translating forecasts into operational shop-floor decisions [4], [5].

Static Tariff Assumptions: Energy-aware scheduling studies overwhelmingly assume that electricity prices, power consumption profiles, or tariff schedules are static, perfectly deterministic, and provided a priori [7], [8].

Consequently, there is an absence of practical, reproducible, end-to-end frameworks that connect industrial energy forecasting, forecast-derived cost profiles, scheduling optimization, and operational KPI evaluation.

E. Research Objectives

To address the identified research gap, this study defines the following specific technical objectives:

1) To preprocess, validate, and clean historical industrial energy-consumption data from a steel manufacturing facility.

2) To engineer comprehensive temporal, cyclical, lagged, and rolling statistical features from industrial energy time series.

3) To develop and evaluate an XGBoost regression model for high-resolution industrial energy-demand forecasting.

4) To generate multi-step energy-demand forecasts at 15-minute resolution across a 24-hour planning horizon (96 intervals).

5) To transform the forecasted energy profile into a time-dependent, peak-weighted scheduling cost profile.

6) To model production jobs, machine compatibilities, power ratings, changeover times, arrival constraints, and delivery deadlines.

7) To develop an FCFS-seeded deterministic energy-aware local-search scheduling algorithm.

8) To perform a comparative benchmark between the baseline FCFS schedule and the energy-aware optimized schedule.

9) To evaluate system performance across modeled energy cost, peak-hour load, makespan, machine utilization, waiting time, tardiness, and on-time completion.

10) To deploy an interactive Streamlit decision-support dashboard for visualizing schedules, forecasts, and comparative KPIs.

F. Major Contributions

The primary contributions of this paper are summarized as follows:

• Integrated Architecture: An end-to-end framework linking machine-learning industrial energy forecasting with production job scheduling.

• Engineered Forecasting Pipeline: An XGBoost forecasting pipeline incorporating temporal, cyclical, autoregressive lag, and rolling statistical features tailored to 15-minute industrial power data.

• Forecast-Driven Cost Modeling: A mathematical formulation converting predicted power demand and peak/off-peak tariff parameters into dynamic time-slot cost coefficients.

• Deterministic Local-Search Optimizer: An FCFS-seeded deterministic local-search heuristic reducing energy costs and peak loads while guaranteeing baseline makespan and deadline compliance.

• Dual-Domain KPI Evaluation: A rigorous comparative analysis evaluating both energy metrics and manufacturing metrics.

• Operational Trade-Off Quantification: Empirical identification of the queue waiting-time penalty incurred when shifting operations for energy efficiency.

• Interactive Decision Dashboard: A multi-view Streamlit dashboard enabling managers to inspect Gantt schedules, examine forecasts, and analyze trade-offs.

• Reproducible Implementation: A modular Python software repository with automated tests and serialized pipelines.

II. RELATED WORK / LITERATURE REVIEW

Energy-aware production scheduling has evolved into a vibrant domain within sustainable manufacturing operations research. Gahm et al. established a foundational taxonomic framework classifying energy-efficient scheduling across energy demand characteristics, energy supply options, and energetic coverage levels, noting that operational scheduling decisions directly modulate industrial power demand [1].

Bänsch et al. conducted a comprehensive systematic review of 192 studies on energy-aware decision-support models in manufacturing, highlighting machine scheduling, lot sizing, and load management as critical areas, while emphasizing that realistic industrial modeling and data-driven decision tools represent continuing research needs [2]. Terbrack et al. surveyed 375 publications on energy-oriented production planning, identifying peak-load management and time-dependent energy pricing as crucial mechanisms for mitigating industrial energy costs without compromising operational throughput [9].

Fernandes et al. reviewed 172 papers on energy-efficient job-shop scheduling, underscoring the dominance of metaheuristics in handling multi-objective trade-offs between energy consumption and makespan, while noting a scarcity of integrated data-driven approaches [6]. Catanzaro et al. provided an extensive survey of job scheduling under Time-of-Use (TOU) electricity tariffs, demonstrating that time-varying electricity prices can be mathematically discretized into slot-based cost coefficients to guide job sequencing while meeting due dates [3].

In recent mathematical programming research, Dunke and Nickel developed exact and matheuristic formulations for energy-aware job-shop scheduling under dynamic power tariffs and power purchase agreements, proving that joint optimization of makespan and electricity cost achieves substantial savings [7]. Fisco-Compte et al. formulated sustainable production scheduling models under implicit demand response, real-time pricing, and carbon emission policies, showcasing how flexible production windows can absorb electricity price volatility [8].

Regarding industrial energy forecasting, Sathishkumar et al. analyzed electrical energy consumption in a Korean steel manufacturing facility using data-mining algorithms, establishing that temporal, electrical, and reactive-power variables accurately predict plant power demand [4], [5]. On the algorithmic front, Chen and Guestrin introduced XGBoost, an extreme gradient boosted decision tree system renowned for handling tabular data, non-linear interactions, and temporal features efficiently [10].

TABLE I
COMPARISON OF RELATED WORKS IN ENERGY-AWARE SCHEDULING AND FORECASTING

| Study | Year | Method | Major Contribution | Identified Gap |
| --- | --- | --- | --- | --- |
| Gahm [1] | 2016 | Review | Energy scheduling framework | Lacks predictive ML integration |
| Bänsch [2] | 2021 | Review | 192 decision models classified | AI & data pipelines needed |
| Terbrack [9] | 2021 | Review | Load management & supply | Lacks slot-level scheduling |
| Fernandes [6] | 2022 | Review | 172 job-shop scheduling papers | Dynamic data pipelines scarce |
| Catanzaro [3] | 2023 | Survey | TOU tariff job scheduling | Assumes static known tariffs |
| Dunke [7] | 2025 | Exact | Dynamic tariffs & PPAs | Requires fixed price matrix |
| Fisco [8] | 2025 | Opt. | Demand response & carbon | Case-specific price curves |
| Sathish [5] | 2021 | ML | Steel plant power prediction | Forecasting without scheduling |
| Proposed | 2026 | Hybrid | Forecast-driven scheduling | Evaluated on synthetic jobs |

As demonstrated in Table I, the literature has historically treated industrial energy forecasting and production scheduling as isolated disciplines. This research directly bridges this gap by establishing an integrated, reproducible data-to-decision pipeline connecting XGBoost energy forecasting, dynamic cost generation, and deterministic energy-aware local-search scheduling.

III. METHODOLOGY

A. Research Design

This study follows an experimental system-development methodology designed for reproducibility and modularity. The research design is structured into five distinct phases: (1) Data validation, cleaning, and multi-scale feature extraction; (2) XGBoost model training and 24-hour recursive forecasting; (3) Cost profile mapping and dual-track scheduling (FCFS vs. Local Search); (4) Multi-criteria KPI evaluation; and (5) Interactive Streamlit dashboard deployment.

B. System Architecture

The overall architecture of the proposed integrated framework is illustrated in Fig. 1. The data flow progresses systematically from raw industrial inputs through validation, feature engineering, predictive modeling, tariff mapping, dual-track scheduling, KPI evaluation, and interactive visual reporting.

[Insert Figure 1 generated from project output here] Data Ingestion → Preprocessing & Feature Engineering → XGBoost 24-Hour Forecast → Forecast-Weighted Cost Profile → Dual Schedulers (FCFS & Local Search) → KPI Evaluation → Streamlit Dashboard

Fig. 1.  Proposed integrated architecture of the forecast-driven energy-aware manufacturing scheduling framework.

C. Dataset Descriptions

1) Industrial Energy Dataset:

The energy forecasting model is developed using the public Steel Industry Energy Consumption Dataset from the UCI Machine Learning Repository [4]. The dataset contains 35,040 continuous observations recorded at 15-minute intervals spanning a complete calendar year (365 days) from DAEWOO Steel Co. Ltd. in Gwangyang, South Korea, monitored via the Korea Electric Power Corporation (KEPCO) system. The target variable is Usage_kWh, representing total active electrical energy consumed during each 15-minute interval. Table II details the 10 raw features.

TABLE II
ATTRIBUTES OF THE UCI STEEL INDUSTRY ENERGY DATASET

| Feature Name | Type | Unit / Domain | Description |
| --- | --- | --- | --- |
| Usage_kWh | Cont. | kWh | Active energy consumed in 15-min (Target) |
| Lagging_Current_Reactive | Cont. | kVARh | Reactive power from inductive loads |
| Leading_Current_Reactive | Cont. | kVARh | Reactive power from capacitive loads |
| CO2 | Cont. | ppm | Estimated plant carbon emissions |
| Lagging_Current_PF | Cont. | % | Power factor for lagging current |
| Leading_Current_PF | Cont. | % | Power factor for leading current |
| NSM | Cont. | Sec (0–86.4k) | Seconds elapsed from midnight |
| WeekStatus | Cat. | Weekday/End | Workday vs. weekend classification |
| Day_of_week | Cat. | Mon–Sun | Day of the week of measurement |
| Load_Type | Cat. | Light/Med/Max | Factory operational load classification |

2) Machine Specification Dataset:

The machine dataset comprises 10 manufacturing machines characterizing a multi-machine shop floor. Because the public steel industry dataset contains plant-level energy measurements without granular machine-level telemetry, synthetic machine specifications were developed to represent realistic manufacturing equipment. Table III outlines the machine attributes.

TABLE III
ATTRIBUTES OF THE MANUFACTURING MACHINE DATASET

| Attribute Name | Type | Unit | Operational Description |
| --- | --- | --- | --- |
| Machine_ID | Str. | ID | Unique identifier (M01 to M10) |
| Machine_Type | Cat. | Type | Functional category (CNC, Lathe, etc.) |
| Idle_Power_kW | Cont. | kW | Standby baseline power consumption |
| Active_Power_kW | Cont. | kW | Active power during processing |
| Changeover_Time | Int. | Min | Setup & tooling preparation duration |
| Available_From | Time | HH:MM | Start of machine availability window |
| Available_To | Time | HH:MM | End of machine availability window |

3) Production Job Dataset:

The job dataset contains approximately 100 synthetic production jobs representing work orders released across the scheduling horizon. Synthetic job data were required because the public energy dataset does not contain individual work order records. Table IV outlines the production job attributes.

TABLE IV
ATTRIBUTES OF THE PRODUCTION JOB DATASET

| Attribute Name | Type | Unit | Operational Description |
| --- | --- | --- | --- |
| Job_ID | Str. | ID | Unique job identifier (J001 to J100) |
| Duration_min | Int. | Min | Nominal processing duration required |
| Deadline | Date | Time | Latest allowable completion time |
| Priority | Cat. | Level | Priority weighting (High/Med/Low) |
| Compatible_M | List | IDs | Machines capable of executing the job |
| Arrival_Time | Date | Time | Earliest release / arrival time |

D. Data Preprocessing and Feature Engineering

Data Validation and Cleaning: Automated validation checks verify dataset dimensions (35,040 rows), missing value counts, duplicate rows, empty columns, and data type correctness. Physical boundary constraints are verified: negative energy consumption values are flagged and removed, and power factor attributes are verified within the range [0, 100]. Timestamps are converted to standard DateTime objects and sorted chronologically. Missing values in continuous electrical series are imputed using forward-fill followed by backward-fill, preserving temporal continuity.

Feature Engineering: Four comprehensive categories of features are extracted from the validated energy time series: (1) Temporal Features (Hour h ∈ [0, 23], Day d ∈ [1, 31], Month m ∈ [1, 12], Day of week w ∈ [0, 6], Quarter q ∈ [1, 4], Day of year d_y ∈ [1, 365], Weekend indicator, and NSM); (2) Cyclical Features preserving periodic continuity as defined in (1) and (2); (3) Autoregressive Lag Features at intervals of 1, 2, 4, and 96 steps (15 min, 30 min, 1 hour, and 24 hours); and (4) Rolling Window Statistics (mean, standard deviation, min, max, first-order differences Δu_t = u_t − u_{t−1}, and EWMA).

| Hoursin = sin2πh24, Hourcos = cos2πh24 | (1) |
| --- | --- |

| Monthsin = sin2π(m-1)12, Monthcos = cos2π(m-1)12 | (2) |
| --- | --- |

E. Industrial Energy-Demand Forecasting Model

Predictive Model Architecture: Extreme Gradient Boosting (XGBoost) regression is selected for energy forecasting [10]. XGBoost constructs an ensemble of gradient-boosted decision trees using regularized objective functions that minimize squared error loss while penalizing model complexity as shown in (3):

| lθ = i=1nyi - yi2 + k=1KΩfk | (3) |
| --- | --- |

where Ω(f_k) = γT_k + (1/2)λ ∑ w_{jk}² penalizes leaf count T_k and leaf weights w_{jk}. Table V details the hyperparameter configuration utilized in the current system implementation.

TABLE V
XGBOOST HYPERPARAMETER CONFIGURATION

| Hyperparameter | Configured Value | Operational Rationale |
| --- | --- | --- |
| n_estimators | 500 | Sufficient iterations for gradient convergence |
| max_depth | 6 | Captures non-linear feature interactions |
| learning_rate | 0.03 | Small shrinkage factor for stable descent |
| subsample | 0.80 | Stochastic row subsampling prevents co-adaptation |
| colsample_bytree | 0.80 | Feature subsampling per split reduces correlation |
| min_child_weight | 3 | Minimum instance weight sum in a leaf |
| objective | reg:squarederror | Minimizes standard squared-error loss |
| random_state | 42 | Ensures strict deterministic reproducibility |

Training and Testing Partitioning: The 35,040 energy observations are partitioned chronologically into 80% training data (first 28,032 observations, ~292 days) and 20% testing data (last 7,008 observations, ~73 days) without random shuffling to prevent look-ahead bias.

Forecasting Horizon & Metrics: The model produces multi-step forecasts across a 24-hour future planning horizon (96 intervals). Model accuracy is evaluated using MAE, RMSE, R², and MAPE, as defined in (4)–(7):

| MAE = 1ni=1nyi - yi | (4) |
| --- | --- |

| RMSE = 1ni=1nyi - yi2 | (5) |
| --- | --- |

| R2 = 1 - i=1nyi - yi2i=1nyi - y2 | (6) |
| --- | --- |

| MAPE = 100ni=1nyi - yiyi | (7) |
| --- | --- |

Forecasting Results: Table VI reports the checked-in experimental performance of the XGBoost model on the chronological test split. These metrics reflect current baseline results and should be verified prior to final industrial deployment.

TABLE VI
XGBOOST ENERGY FORECASTING PERFORMANCE

| Evaluation Metric | Measured Value | Unit / Interpretation |
| --- | --- | --- |
| Mean Absolute Error (MAE) | 0.5078 | kWh (Mean absolute error) |
| Root Mean Squared Error (RMSE) | 0.9976 | kWh (Penalizes large deviations) |
| Coefficient of Determination (R²) | 0.9990 | Normalized variance explained |
| Mean Absolute Percentage Error (MAPE) | 2.4718% | Percentage error relative to demand |

F. Mathematical Formulation and Scheduling Algorithms

1) Time Discretization and Notation:

The scheduling horizon is discretized into uniform 15-minute slots t ∈ {1, 2, ..., T}. For a production job j with duration D_j (minutes), the integer slot requirement P_j is defined in (8):

| Pj = Dj15 | (8) |
| --- | --- |

Changeover duration c_m on machine m is similarly mapped to integer slots.

2) Operational Constraints:

The scheduling problem is governed by four core operational constraints:

• Compatibility: Job j must be assigned to exactly one compatible machine m ∈ M_j, as formulated in (9):

| m∈Mjxjm = 1, ∀j ∈ J, xjm ∈ {0, 1} | (9) |
| --- | --- |

• Non-Overlap: Jobs i and j on machine m must not overlap, accounting for changeover c_{jm}, as expressed in (10):

| si ≥ ej + cjm or sj ≥ ei + cim | (10) |
| --- | --- |

• Release Time: Job j cannot start before arrival slot a_j, as defined in (11):

| sj ≥ aj, ∀j ∈ J | (11) |
| --- | --- |

• Tardiness & Delay: Tardiness D_j against deadline d_j is defined in (12). The system strictly enforces zero tardiness (∑ D_j = 0):

| Dj = max0, ej - dj | (12) |
| --- | --- |

3) Energy Consumption and Cost Model:

For machine m with active power rating P_m (kW), electrical energy E_{jm} and modeled energy cost C_j are formulated in (13) and (14):

| Ejm = Pm × 1560 × Pj (kWh) | (13) |
| --- | --- |

| Cj = t=sjej - 1Pm1560rt | (14) |
| --- | --- |

where effective rate r_t is defined in (15) across peak (08:00–20:00, 0.25) and off-peak (0.10) periods based on forecasted energy demand û_t:

| rt = 0.25 · ut, t ∈ Peak (08:00–20:00)0.10 · ut, t ∈ Off-Peak (20:00–08:00) | (15) |
| --- | --- |

4) Baseline First-Come-First-Served (FCFS) Heuristic:

The FCFS heuristic sorts jobs by arrival time a_j (priority tie-breaker) and greedily assigns each job to the earliest available compatible machine m ∈ M_j, including changeovers c_m, recording baseline completion times and energy costs.

5) Proposed Energy-Aware Deterministic Local Search:

The active scheduler initializes with the FCFS schedule, identifies candidate jobs executing in expensive peak windows (08:00–20:00), explores alternative compatible machines and start slots s'_j ≥ a_j, prunes candidates violating non-overlap, baseline makespan C_max^{FCFS}, deadlines (D_j = 0), or peak load, and greedily accepts shifts yielding strictly lower modeled energy costs until convergence.

6) CP-SAT Formulation Distinction:

The codebase contains a separate Google OR-Tools CP-SAT formulation. However, CP-SAT is not invoked by the active optimizer. All reported results are generated strictly by the FCFS-seeded deterministic local search.

G. Experimental Setup and Configuration

Experimental parameters are summarized in Table VII. Implementation is in Python 3 using Pandas, NumPy, Scikit-learn, XGBoost, Joblib, OR-Tools, Streamlit, Plotly, Matplotlib, OpenPyXL, and Pytest. Hardware configuration: [Hardware configuration to be added].

TABLE VII
EXPERIMENTAL PARAMETER CONFIGURATION

| Parameter | Value / Configuration |
| --- | --- |
| Total Energy Records | 35,040 observations (1 full year) |
| Sampling Resolution | 15-minute intervals |
| Target Variable | Usage_kWh |
| Data Split | 80% Train (28,032) / 20% Test (7,008), Chronological |
| Forecasting Horizon | 24 hours (96 intervals) |
| Scheduling Slot | 15 minutes per slot |
| Machines / Jobs | 10 machines / ~100 synthetic production jobs |
| Schedulers | FCFS Baseline vs. Deterministic Local Search |
| Tariff Model | Peak: 08:00–20:00 (0.25) / Off-Peak: 0.10 |
| Random Seed | 42 (Strict deterministic repeatability) |

IV. RESULTS AND DISCUSSION

A. Experimental Results

Table VIII presents the quantitative comparison between FCFS and the energy-aware optimized schedule across all evaluated Key Performance Indicators (KPIs).

TABLE VIII
COMPREHENSIVE PERFORMANCE COMPARISON: FCFS VS. ENERGY-AWARE OPTIMIZED SCHEDULE

| Key Performance Indicator | FCFS | Optimized | Impact |
| --- | --- | --- | --- |
| Total Modeled Energy Cost | 1713.88 | 1443.78 | 15.8% Reduction |
| Peak-Hour Load (kWh) | 1605.83 | 1141.67 | 28.9% Reduction |
| Makespan (Minutes / Hours) | 1350 / 22.5 | 1350 / 22.5 | Maintained |
| Average Machine Utilization | 57.92% | 57.92% | Maintained |
| Average Waiting Time (Min) | 141.9 | 241.5 | +99.6 min (+70.2%) |
| Total Delay / Late Jobs | 0 min / 0 | 0 min / 0 | Zero Tardiness |
| On-Time Completion Rate | 100% | 100% | 100% On-Time |

1) Modeled Energy Cost Reduction:

The energy-aware scheduler reduces modeled energy cost from 1713.88 to 1443.78, achieving a relative reduction of 15.8% as calculated in (16):

| ΔCost = 1713.88 - 1443.781713.88 × 100% = 15.8% | (16) |
| --- | --- |

2) Peak-Hour Load Shaving:

Peak-hour power demand (08:00–20:00) decreases from 1605.83 kWh to 1141.67 kWh (28.9% reduction) as calculated in (17), mitigating grid-level demand spikes:

| ΔPeakLoad = 1605.83 - 1141.671605.83 × 100% = 28.9% | (17) |
| --- | --- |

3) Manufacturing Throughput and Makespan Preservation:

Factory throughput is preserved identically: makespan remains 1350 minutes (22.5 hours), machine utilization remains 57.92%, total delay remains 0 minutes, and on-time completion is 100% (0 late jobs).

4) The Waiting-Time Trade-Off:

Average job waiting time rises from 141.9 minutes under FCFS to 241.5 minutes under the energy-aware schedule—an increase of 99.6 minutes (+70.2%) as expressed in (18):

| ΔWait = 241.5 - 141.9 = +99.6 min | (18) |
| --- | --- |

This illustrates the core trade-off: flexible jobs are intentionally delayed within allowable slack to avoid expensive peak intervals, trading queue latency for substantial energy-cost savings.

B. Comprehensive Discussion

Physical Mechanism: Under FCFS, jobs execute immediately upon arrival, concentrating heavy processing during peak midday hours. In contrast, the local-search optimizer shifts flexible operations to off-peak slots (before 08:00 or after 20:00), cutting cost by 15.8% and peak load by 28.9%.

Multi-Objective Dynamics: Energy cost, peak electrical load, waiting time, makespan, machine utilization, and tardiness compete directly. Traditional heuristics prioritize queue velocity (minimizing waiting time), whereas energy-aware algorithms exploit queue slack to minimize power cost. This finding aligns with observations in broader scheduling literature [6], [7].

Methodological Consideration: In the forecasting pipeline, features such as CO2 emissions and reactive power reflect contemporaneous electrical measurements. In live deployments, future contemporaneous values may not be known in advance; hence, a feature-leakage audit should evaluate restricted feature sets relying strictly on lagged autoregressive and calendar variables.

1) Decision Support Dashboard and Visualization:

A modular Streamlit decision dashboard was developed comprising: (1) Overview KPIs, (2) Interactive Gantt scheduling, (3) Energy demand forecasting curves and XGBoost feature importance, (4) KPI Benchmarking, and (5) Data export capabilities.

[Insert Figure 2 generated from project output here] Annual 15-minute active energy consumption (Usage_kWh) time series.

Fig. 2.  Historical industrial energy-consumption profile from the UCI Steel Industry dataset.

[Insert Figure 3 generated from project output here] XGBoost regression predictions tracking actual power demand.

Fig. 3.  Actual versus predicted energy consumption on the chronological test split.

[Insert Figure 4 generated from project output here] Zero-centered Gaussian error histogram illustrating unbiased residuals.

Fig. 4.  Residual error distribution of the XGBoost forecasting model.

[Insert Figure 5 generated from project output here] Recursive multi-step forecast curve highlighting peak and off-peak load regimes.

Fig. 5.  Future 24-hour industrial energy-demand forecast containing 96 15-minute intervals.

[Insert Figure 6 generated from project output here] Gini importance ranking showing dominance of autoregressive lags.

Fig. 6.  Feature importance ranking of the XGBoost energy forecasting model.

[Insert Figure 7 generated from project output here] Timeline showing heavy job clustering during peak tariff hours.

Fig. 7.  Gantt chart of the baseline First-Come-First-Served (FCFS) production schedule.

[Insert Figure 8 generated from project output here] Optimized timeline showing strategic shifting to off-peak windows.

Fig. 8.  Gantt chart of the proposed energy-aware production schedule.

[Insert Figure 9 generated from project output here] Bar chart comparing energy cost, peak load, makespan, and waiting time.

Fig. 9.  Multi-criteria KPI benchmark comparison between FCFS and energy-aware schedules.

[Insert Figure 10 generated from project output here] Superimposed energy curves illustrating peak shaving and valley filling.

Fig. 10.  Time-slot energy-cost profile showing redistribution of load away from peak tariff windows.

C. Limitations

Academic integrity requires acknowledging current limitations: (1) Synthetic job/machine data; (2) Forecast-weighted modeled cost function rather than complex utility invoices; (3) Deterministic local-search heuristic rather than exact global optimality; (4) Machine availability windows not fully enforced; (5) Machine idle power not incorporated into optimization; (6) Recursive multi-step forecasting approximations; (7) Contemporaneous feature leakage risk; (8) Single industrial dataset evaluated; and (9) Absence of live SCADA/MES factory integration.

V. CONCLUSION AND FUTURE WORK

This research presented an integrated framework coupling industrial energy forecasting with energy-aware machine scheduling. An XGBoost regression model trained on steel industry energy data achieved an MAE of 0.5078 kWh and an R² of 0.999 across a 24-hour horizon. The resulting forecast guided an FCFS-seeded deterministic local-search scheduler, reducing modeled energy cost by 15.8% and peak-hour load by 28.9%, while maintaining makespan (22.5 hours), machine utilization (57.92%), and 100% on-time completion. Concurrently, average waiting time increased by 99.6 minutes (+70.2%), quantifying the fundamental trade-off between energy optimization and queue latency.

A. Future Research Directions

Future work will: (1) Connect the existing CP-SAT exact formulation to the active execution path; (2) Implement Pareto multi-objective metaheuristics (NSGA-II); (3) Benchmark deep learning models (LSTM, GRU, TFT); (4) Calibrate with real-world dynamic electricity tariffs; (5) Model machine idle power dynamics; (6) Validate on empirical factory dispatch logs; and (7) Perform systematic sensitivity and robustness analyses.

ACKNOWLEDGMENT

The authors thank the Department of Artificial Intelligence and Data Science, Rajalakshmi Engineering College, Chennai, India, for academic facilities and guidance. The authors also acknowledge the UCI Machine Learning Repository and DAEWOO Steel Co. Ltd. for the open dataset.

DECLARATIONS AND STATEMENTS

Data Availability: The UCI Steel Industry dataset is available under CC BY 4.0 (DOI: 10.24432/C52G8C) [4]. Synthetic datasets are available upon request.

Code Availability: Complete Python codebase is available at: [Insert Repository URL].

Ethics & Conflicts: No human/animal subjects involved. The authors declare no competing interests.

Author Contributions: Bijlesh S: Conceptualization, Methodology, Software, Data Curation, Formal Analysis, Writing – Original Draft. Balaji Ramesh: Data Preprocessing, Feature Engineering, Software, Validation, Writing – Review & Editing. Hemanth Kumar D: Scheduling Optimization, Experimental Design, Performance Benchmarking, Writing – Review & Editing.

REFERENCES

[1] C. Gahm, F. Denz, M. Dirr, and A. Tuma, “Energy-efficient scheduling in manufacturing companies: A review and research framework,” European Journal of Operational Research, vol. 248, no. 3, pp. 744–757, 2016, doi: 10.1016/j.ejor.2015.07.017.

[2] K. Bänsch, J. Busse, F. Meisel, J. Rieck, S. Scholz, T. Volling, and M. G. Wichmann, “Energy-aware decision support models in production environments: A systematic literature review,” Computers & Industrial Engineering, vol. 159, Art. no. 107456, 2021, doi: 10.1016/j.cie.2021.107456.

[3] D. Catanzaro, R. Pesenti, and R. Ronco, “Job scheduling under Time-of-Use energy tariffs for sustainable manufacturing: A survey,” European Journal of Operational Research, vol. 308, no. 3, pp. 1091–1109, 2023, doi: 10.1016/j.ejor.2023.01.029.

[4] S. V. E. Sathishkumar, C. Shin, and Y. Cho, “Steel Industry Energy Consumption,” UCI Machine Learning Repository, 2021, doi: 10.24432/C52G8C.

[5] S. V. E. Sathishkumar, C. Shin, and Y. Cho, “Efficient energy consumption prediction model for a data analytic-enabled industry building in a smart city,” Building Research & Information, vol. 49, no. 1, pp. 127–143, 2021, doi: 10.1080/09613218.2020.1809983.

[6] J. M. R. C. Fernandes, S. M. Homayouni, and D. B. M. M. Fontes, “Energy-Efficient Scheduling in Job Shop Manufacturing Systems: A Literature Review,” Sustainability, vol. 14, no. 10, Art. no. 6264, 2022, doi: 10.3390/su14106264.

[7] F. Dunke and S. Nickel, “Approximate and exact approaches to energy-aware job shop scheduling with dynamic energy tariffs and power purchase agreements,” Applied Energy, vol. 380, Art. no. 125065, 2025, doi: 10.1016/j.apenergy.2024.125065.

[8] P. Fisco-Compte, M. Juanpera, B. Domenech, R. Pastor, M. Ranaboldo, and E. Bullich-Massagué, “Sustainable optimal production scheduling methodology under implicit demand response and carbon policy: A manufacturing industry case study,” Applied Energy, vol. 384, Art. no. 125403, 2025, doi: 10.1016/j.apenergy.2025.125403.

[9] H. Terbrack, T. Claus, and F. Herrmann, “Energy-Oriented Production Planning in Industry: A Systematic Literature Review and Classification Scheme,” Sustainability, vol. 13, no. 23, Art. no. 13317, 2021, doi: 10.3390/su132313317.

[10] T. Chen and C. Guestrin, “XGBoost: A scalable tree boosting system,” in Proc. 22nd ACM SIGKDD Int. Conf. Knowledge Discovery and Data Mining, 2016, pp. 785–794, doi: 10.1145/2939672.2939785.